| Purpose | Tāngata whaiora/tāngata whaikaha, workers (employees, volunteers, contractors), and people spending time at premises we own, lease, rent or occupy for providing a service, are provided with a suitable and accessible physical environment. All premises are fit for their purpose. |  |
| --- | --- | --- |
| Scope | All buildings, grounds and places our organisation uses for service delivery and administrative purposes. |  |
| References |  |  |
| Legislation | The Building Act 2004 |  |
| Guidelines | Building Performance NZ Govt. Building Warrant of Fitness Code of Compliance Energy safety Guide to hot water cylinder temperatures GENLESS NZ Govt. |  |
| Organisation’s policies/ procedures | Asset Management Emergency and Security Management Health and Safety Infection Control and Antimicrobial Stewardship Smoke-free Environment |  |
| Buildings and Property |  |  |
| Policy |  | - We comply with the relevant building codes and other requirements. - We maintain a Building Warrant of Fitness (BOF) or Code of Compliance Certificate (CCC) or provide evidence that a BOF or CCC is not required. - The people living and working at the premises are involved in decisions about how spaces are used and set up. - Mana whenua are consulted in order to provide a culturally enhanced living spaces. - We provide adequate spaces for the people who live and work at the premises. |
| Premises |  |  |
| Responsibility: Our governing body |  |  |
| We ensure that the premises we operate from: - Can be easily used by people with disabilities or impairments. For example: Sensory cues, ramps, level-access shower, even paths, adequate task and general lighting, dimmer switches for low vision, handrails, and other supporting features as required. - Are culturally safe. For example: laundry equipment and facilities are not in the kitchen or anywhere else where food is handled; the blessing of the property to lift a tapu. - Enable infection control practices. For example: - Kitchen, bathrooms, toilets, laundry floors and walls are made from material that can be cleaned in line with infection prevention and control practices. Example materials: Tiles, plastic, metal, formica, acrylic, ceramic, lino. - Soft furnishings that are easy to clean. - Have non-slip surfaces or other safe, effective means of minimising slipping in areas frequently exposed to moisture or slippery substances. - The rooms in the building have efficient and adequate heating and cooling systems in place. |  |  |

| Ventilation and heating | Recommended temperatures (AI Google) |
| --- | --- |
| Ventilation guide Heating and cooling | Living areas (winter): 18–21°C. Bedrooms (overnight): 18°C is recommended for sleeping. Vulnerable households: For homes with babies, elderly people, or those who are unwell, maintain occupied rooms at around 20°C. Summer: During summer, 19–21°C is a comfortable and energy-efficient range for general use. For air conditioning, aim for 23–26°C for efficiency. |
| Equipment |  |
| Responsibility: Operational manager and health and safety lead |  |
| - Equipment we use is purchased from reliable sources. - Only registered trades people will install/repair our equipment. - Tāngata whaiora/tāngata whaikaha and workers are instructed to equipment they will be using. For example, how and when to use a ladder, lawn mower, general and specific cleaning equipment and materials, cooking equipment. - Equipment will be calibrated in line with the maintenance log. For example, scales, blood pressure monitor. - The buildings are furnished and equipped of a high standard. - Electrical and gas appliances are regularly checked. This includes heaters and electrical appliances. For example: water kettle, toaster, washing machine. We keep a record of all maintenance activities. |  |
| External areas |  |
| - We maintain our gardens and shrubs and trees. - We avoid to plant allergy-producing plants. - We consider sensory stimulation and aids – for example, chimes, use of colour contrast for low vision and fragrance gardening – to help tāngata whaiora/tāngata whaikaha to negotiate the external environment. - We make outside spaces accessible for all the people at the premises, including through ground surface indicators and non-slip surfaces. - We provide shade in the outside space. - Decks, paths and steps are safe and of a non-slippery surface. |  |
| Shared spaces |  |
| Toilets |  |
| - Are accessible to tāngata whaiora/tāngata whaikaha at all times. - Can be locked. - Can be accessed by workers in case of an emergency. - Have a hand wash basin. - There is a toilet that is accessible to tāngata whaiora/tāngata whaikaha and workers requiring mobility aids. - Gender-neutral toilets are available in shared spaces. - There are sufficient toilets available. We use the toilet calculator to check the number of toilets recommended. |  |
| Shower |  |
| Enough showers are available to the people engaging with our services. Shower rooms: - Can be locked. - Can be accessed by staff in case of an emergency. - Have a non-slip floor. - One shower is accessible to tāngata whaiora/tāngata whaikaha requiring mobility aids. |  |
| Workers are mindful of the possibility that toilets and showers can trigger past trauma experiences for tāngata whaiora/tāngata whaikaha. |  |
| Future rentals or building purchases will include enough toilets and showers so one can be designated specifically for infection isolation purposes. |  |
| Wash basins |  |
| Wash basins are available at: - each toilet - kitchen - laundry - where medication is administered |  |
| Laundry |  |
| Our housing and recovery services have a dedicated laundry space that includes: - washing machine - dryer - outside and/or inside clothing line - separate clean and dirty washing baskets or area |  |
| Note: Water temperatures are: - 45°-50° at the tap/shower head - monitored monthly – a record will be maintained. - Hot water cylinders must be set at a temperature of not less than 60°C to prevent the growth of legionella bacteria. |  |
| Storage for hazardous substances |  |
| - A dedicated lockable area/cupboard is used to store hazardous substances. - This is accessible to workers and specified tangata whaiora/tangata whaikaha. |  |
| Other shared spaces |  |
| Are available for: - cooking - dining - recreation - meditation/contemplation/prayer - work - sports/exercises - sensory modulation |  |
| Personal space |  |
| Bedrooms |  |
| - All bedrooms are single – unless a clinical justification indicates otherwise and tangata whaiora/ tangata whaikaha agrees to such an arrangement. - Windows have blinds or curtains that trap air to block heat or cold (cellular shades or thermal curtains or both). - Windows can be opened to let fresh air in. In some services there might be security latches for safety reasons. - Tāngata whaiora/tāngata whaikaha have access to the bedroom at all times. - Tāngata whaiora/tāngata whaikaha have the opportunity to store their possessions safely (examples: lock their bedroom door, have a lockable cabinet). - Tāngata whaiora/tāngata whaikaha using mobility aids are able to move safely around their personal space/bed area. - Bedrooms have at least one window that provides natural light. |  |

Building and Property Maintenance

Responsibility: Click here to enter text.

Routine maintenance

- Is carried out according to the maintenance log. - An approved suppliers contact list and the nature of their service is maintained.

Repairs

- Workers contact Click here to enter text. once the need for repairs is identified. - Tāngata whaiora/tāngata whaikaha contact the Click here to enter text. - The approved suppliers will be contacted to arrange the repairs being done. - The approved suppliers list is available on Click here to enter text. - A record of the repair request and completion of the repair is maintained.

Urgent repairs

- Workers or tāngata whaiora/tāngata whaikaha inform staff on duty. - Staff on duty contact the approved supplier to arrange an urgent repair. - A record of this is maintained.

- Workers are familiar with the location of the fuse box, water main and hot water cupboards and know how and when to turn them on and off. - Any maintenance and repair issues that are health and safety risks are reported to our Health and Safety Officer. - If the issue is a health and safety issue the Health and Safety management processes are followed. Please refer to the Health and Safety document.